ChronoBloom Compact Function Inventory

v2.2.0  |  139 functions  |  src/main.cpp + src/web_html.h

Source: FUNCTION_INVENTORY.md (auto-generated by tools/gen_symmap.py). Print-friendly edit map. Common targets only; internals condensed at bottom.

Page 1 — Common / Reasonably Likely Edit Targets

| Function / Structure | Telegraph meaning | Edit when |
| --- | --- | --- |
| SettingsStore::defaults | Factory defaults | Default palette, brightness, feature defaults. |
| SettingsStore::valid | EEPROM accept rules | Change when fields/ranges change. |
| SettingsStore::sanitize | Clamp user input | Safety bounds for saved settings. |
| SettingsStore::resetToDefaults | Wipe EEPROM to defaults | Factory reset path; called on version mismatch. |
| ClockSettings | Saved settings schema | Add field -> bump SETTINGS_VERSION. |
| RingConfig | Physical LED map | Ring start index, count, direction. |
| ClockRenderer::render | Full frame pipeline | Overall draw order/priority. |
| ClockRenderer::renderFace | Static ring backdrop | Markers, filler, base colors. |
| ClockRenderer::renderSeconds | Second hand logic | Trail/progress/dot behavior. |
| ClockRenderer::renderMinutes | Minute dot | Minute placement/color behavior. |
| ClockRenderer::renderHours | Hour hand logic | 12h mapping, half-hour advance. |
| ClockRenderer::effectiveBrightness | Brightness selector | Manual/lux/scheduled dimming. |
| ClockRenderer::nightActive | Night window test | Scheduled dimming hours. |
| ClockRenderer::setRingPixel | Logical -> physical LED | Rotation, offset, direction fixes. |
| ClockRenderer::setCenterPixel | Center LED writer | Inline vs separate center output. |
| ClockRenderer::setSacrificialPixelDark | Keep buffer LED black | Sacrificial first LED handling. |
| ClockRenderer::renderCenterIdle | Center breathing pulse | Idle center animation style. |
| ClockRenderer::renderStatus | Status spinner/pulse | WiFi/save/error visual feedback. |
| ClockRenderer::renderHourlyChime | Top-hour sweep | Hourly chime visual style. |
| ClockRenderer::tickAnimation | Animation dispatcher | Routes to per-case anim* method. |
| ClockRenderer::paletteColor | Palette-mapped color | 8-palette system; add palette here. |
| ClockRenderer::triggerAnimDirect | Fire animation by ID | Used by /previewAnimation endpoint. |
| ClockRenderer::triggerReminderDirectAnimation | Fire reminder anim by ID | Reminder palette, warm tones. |
| triggerQuarter/Half/HourAnimation | Start selected animation | Selector mapping and no-op rules. |
| LuxSensor::lux | Read + smooth lux | Gain thresholds, EMA, settle time. |
| LuxSensor::autoBrightnessCached | Lux -> brightness ramp | Curve, min/max feel, ramp speed. |
| LuxSensor::setLuxOverride / clearLuxOverride | Bypass hardware lux | Used by Demo Mode for brightness sim. |
| TimeSync::begin | NTP/TZ startup | Timezone and NTP servers. |
| TimeSync::syncNow | Apply valid NTP time | Epoch guard and localtime conversion. |
| TimeSync::loop | Periodic resync | Callback + 6-hour refresh behavior. |
| TimeModel::set / addMinutes / addHours | Manual time edits | Browser time buttons and sync. |
| ButtonInput::poll | Physical button state machine | Debounce, hold-to-repeat, GPIO pins. |
| ButtonInput::consumeUp / consumeDown | Button delta consumer | Returns +/- minute delta to loop(). |
| DemoMode::loop | Demo sequence driver | Step table, timing, lux override calls. |
| DemoMode::statusJson | Demo status API | JSON payload for /demo/status endpoint. |
| WebUi::begin | WiFi/web bringup | AP fallback, mDNS, OTA, NTP order. |
| WebUi::setupRoutes | HTTP route table | Add/change browser endpoints. |
| WebUi::settingsJson | Settings API output | Expose new settings to UI/JS. |
| WebUi::parseColor | Hex color parser | Color input validation behavior. |
| WebUi::clampByte / clampWord | POST bounds guards | Form min/max safety. |
| FocusReminderScheduler::checkAndFire | Reminder gatekeeper | Days, hours, interval firing. |
| FocusReminderScheduler::getDayOfWeek | Weekday source | Needs pre-NTP epoch guard. |
| FocusReminderScheduler::triggerReminderAnimation | Reminder anim map | Map options to effects. |
| setup | Boot sequence | Add subsystem init here. |
| loop | Main scheduler | Tick, web, render, button, demo. |
| logRuntimeStatus | Serial health dump | Debug fields and cadence. |
| src/web_html.h | PROGMEM HTML/JS payload | Edit UI layout, labels, JS, CSS. Not setupRoutes. |

Rules That Prevent Pain

| Rule | Why it matters |
| --- | --- |
| EEPROM rule | Changing ClockSettings layout requires SETTINGS_VERSION bump or stale EEPROM will bite. |
| Rotation rule | outerRingOffset is one outer-ring step; if one step worse, reverse direction. |
| Sacrificial LED rule | Do not remove setSacrificialPixelDark; buffer LED must stay black. |
| UI setting rule | New setting touches ClockSettings, defaults, valid, sanitize, settingsJson, setupRoutes, web_html.h, version bump. |
| Animation rule | Visual look lives in anim* methods; dispatcher is tickAnimation; selector start is trigger*Animation. |
| HTML rule | All web UI HTML/JS lives in src/web_html.h. Page content changes go there, not to setupRoutes. |
| Demo rule | Demo step table and timing live in DemoMode::loop. Lux simulation uses setLuxOverride/clearLuxOverride. |
| Button rule | ButtonInput uses polled reads (no ISRs). GPIO5=UP, GPIO9=DOWN. Hold >500ms repeats; >2s jumps 60min. |

Page 2 — Rarely Edited / Support Internals

Real functions, but normally not the first place to edit unless debugging plumbing, boot, or timing.

- configureWiFiHostname – hostname plumbing before DHCP

- writeStatusLed – GPIO LED helper; no-op when STATUS_LED_PIN < 0

- setupWiFi – usually edit credentials/build flags, not logic

- setupOTA – OTA handlers/password/log text

- setupDemoModeRoutes – registers /demo/* routes at runtime; avoids incomplete-type issues

- LuxSensor::begin – VEML7700 init only

- LuxSensor::autoBrightness / autoBrightnessTarget - wrapper and diagnostics only

- SettingsStore::begin / update / save - EEPROM load/save wrappers

- TimeModel::get / tickOneSecond / consumeDirty / incrementSecondNoLock / markDirty - clock plumbing

- ClockRenderer::ClockRenderer – constructor

- ClockRenderer::begin – strip init/clear/show

- ClockRenderer::setLuxSensor / setStatus / animating - wiring setters and state flag

- ClockRenderer::renderAnimFrame / renderCenterAnimationFrame - animation render wrappers

- ClockRenderer::needsFullAnimationFrame / needsCenterAnimationFrame - 80ms render gates

- ClockRenderer::dim / scale / pulse / scaledElapsed / secondColor / ringColor - color/timing math helpers

- ClockRenderer::chimeActive / statusActive / centerIdleActive - small boolean gates

- ClockRenderer::logShow – serial LED timing diagnostics

- ClockRenderer::animQ1-Q6 – 6 quarter-hour animation implementations

- ClockRenderer::animH1-H7 – 7 half-hour animation implementations

- ClockRenderer::animHr1-Hr10 – 10 top-of-hour animation implementations

- ClockRenderer::animRem1-Rem6 – 6 focus reminder animation implementations (edit specific method for that effect)

- ButtonInput::begin / consumeUpPress / consumeDownPress - button init and raw press consumers

- DemoMode::DemoMode / start / stop - demo lifecycle; step table is in loop()

- WebUi::loop – handleClient + ArduinoOTA pump

- WebUi::boolJson / colorHex / hexNibble / wifiStatusText / setLuxSensor / enabled / apMode - output helpers

- TimeSync::TimeSync / synced / lastDeltaSec - constructor and state accessors

- TemperatureInput – disabled stub in current builds

- ClockTime – h/m/s POD copy

- AnimPhase / StatusMode - enum IDs for animations/status

Fast Edit Path Cheat Sheet

| Scenario | Go to |
| --- | --- |
| Color/default look | SettingsStore::defaults, renderFace, hand renderers |
| Clock physically rotated | setRingPixel, RingConfig, outerRingOffset |
| Sacrificial WS2812B | setSacrificialPixelDark, ring offsets/config, pixel count |
| Web UI page content (HTML/CSS/JS) | src/web_html.h -- not setupRoutes |
| Web UI endpoints/logic | setupRoutes, settingsJson, clamps/sanitize |
| New saved option | ClockSettings, defaults, valid, sanitize, settingsJson, POST handler, web_html.h, version bump |
| Brightness feels wrong | effectiveBrightness, LuxSensor::lux, autoBrightnessCached |
| Reminder weirdness | checkAndFire, getDayOfWeek, triggerReminderAnimation |
| Physical buttons | ButtonInput::poll, consumeUp/consumeDown. Pins: GPIO5=UP, GPIO9=DOWN |
| Demo mode sequence or timing | DemoMode::loop (step table), DemoMode::statusJson |
| Specific animation look | ClockRenderer::animQ/H/Hr/Rem* -- find the matching method |