ChronoBloom

An analog LED clock that sees your room, knows your schedule, and interrupts your hyperfocus.

Built from scratch. Coded alone. Running beautifully on a $5 chip.

| 3 LED rings 60 / 24 / 12 LEDs | 17 Animations across 4 trigger types | 8 Color palettes fully customizable | 2,200+ Firmware lines single file, 102 functions | 13+ Dev sessions v2.0 through v2.1 |
| --- | --- | --- | --- | --- |

| What it is Three concentric NeoPixel rings spin light into an analog clock face. No display. No text. Just color, position, and glow — the way a real clock should feel. The outer ring (60 LEDs) tracks seconds with a fading trail and progress arc. The middle ring (24 LEDs) marks minutes with quarter-hour accents. The inner ring (12 LEDs) holds the hour hand. A center pixel breathes quietly at idle. Underneath, a Seeed XIAO ESP32-C3 the size of a thumb drive runs the entire show: NTP-synced time, a full web interface, over-the-air firmware updates, and a VEML7700 ambient light sensor that watches the room. Connect once. Forget forever. First boot opens a captive Wi-Fi portal on your phone. Pick your network, enter the password, done. Every boot after that is automatic. If the network disappears, the clock falls back to its own access point at 192.168.4.1 and keeps running. After first setup, everything is managed from a browser — no app, no USB cable, no re-flashing. The web UI is served directly from the clock at esp32c3-v3-8inch.local. |  | Built for ADHD brains Hyperfocus is real. You sit down to "quickly check something" and surface 3 hours later. ChronoBloom has a dedicated system for this. Focus Reminders - Set an interval — every 25 minutes, every hour, whatever works - Define active hours so it only nudges during your work window - Pick which days of the week it runs - Choose the animation: a quick pulse, a sweeping wave, or a full chime - Survives reboots — saved to permanent storage When the reminder fires, the rings light up in a distinctive pattern that cuts through peripheral vision without being obnoxious. It does not beep. It does not vibrate. It does not send a notification. It is just light, doing its job. Scheduled down to the day Sunday afternoon, Tuesday morning, weekdays only — a days-of-week bitmask means the reminder follows your actual schedule, not a generic repeat. |
| --- | --- | --- |

| ✦ 17 Animations Quarter-hour pulse at :15 Half-hour sweep at :30 Full chime cascade at :00 5 top-of-hour variants Sparkle, tidal pulse, rainbow spiral Breathing mandala, zenith cascade Firework burst | ◈ Smart Brightness VEML7700 lux sensor watches the room Logarithmic brightness curve 3-state gain with hysteresis Smooth 5-second ramp, no snapping Night schedule by hour Day / Auto / Schedule modes | ⬡ Web UI Live SVG clock preview (90ms refresh) Per-ring RGB + brightness sliders 8 color themes + custom NTP sync, browser sync Animation on/off toggles OTA firmware update page Network diagnostics (/diag) |
| --- | --- | --- |

The Build

| By the numbers 13 Firmware sessions Claude Code + Chat; 11+ Changelog entries v2.0.0 through v2.1. 102 Functions fully inventoried; v11 EEPROM settings 256 bytes, versioned. Every function is documented in a machine-readable symbol map (symmap.json). Every change has a CHANGELOG entry with a restore path. Every session has a handoff prompt so work can resume without losing context. RAM usage sits at 11.1% (36 KB / 320 KB). Flash at 56.3%. The ESP32-C3 runs the web server, NTP sync, animation engine, lux sensor polling, OTA receiver, mDNS, and focus reminder scheduler — all non-blocking in a single loop. What it took to get here - Full Wi-Fi provisioning stack with AP fallback and mDNS auto-reconnect - VEML7700 gain oscillation debugged and fixed with hysteresis state machine - GPIO3/4 JTAG conflict discovered and worked around (spurious button fires) - Brightness snapping eliminated with 50-unit/sec ramp function - Sacrificial LED level-shift for 3.3V ESP32 driving 5V WS2812B chain - Day-of-week NTP integration after localtime_r() fix - OTA via ArduinoOTA and browser-based firmware upload page - Live SVG preview rendered in browser from device state every 90ms - Animation engine: non-blocking millis() state machines throughout - Session handoff discipline: PRIMER.md, SESSIONS.md, WORKFLOW.md, REVIEW.md |  | The lineage ChronoBloom did not start here. It started in 2015 when Steve Manley built a 3-ring NeoPixel clock on an Arduino Nano with a DS3234 RTC and a laser-cut case. In 2022, Maestro8484 started building from Steve’s design and eventually rewrote everything for the ESP32. What began as a Wi-Fi upgrade became a full rewrite: new hardware, new rendering engine, new web UI, sensor integration, and eventually — a feature Steve’s original had no reason to include. The Focus Reminder system. Because ADHD does not care about your clock. Hardware MCU; XIAO ESP32-C3. Outer ring; 60 LED WS2812B — seconds. Middle ring; 24 LED WS2812B — minutes. Inner ring; 12 LED WS2812B — hours. Center pixel; index 97 (8") / GPIO20 (15"). Light sensor; VEML7700 I2C — auto-brightness. Data line; GPIO10 + 300Ω + sacrificial LED. Power; 5V rail (LEDs), 3.3V (logic). Variants; 8" and 15" enclosure sizes. What's next - Demo video with live subtitle overlays (OBS + /demo/status endpoint) - Open source release: GitHub + Printables (8" STLs) + Hackaday.io - Holiday auto-animations triggered by calendar date - Multiple Focus Reminder rules (3–5 concurrent schedules) - Pomodoro timer mode on outer ring |
| --- | --- | --- |

ChronoBloom ESP32-C3  •  v2.8.0  •  Apache 2.0 firmware / CC BY 4.0 hardware  •  Maestro8484  •  2022–2026